Wireless Network Security Implementation Report

Date: March 17, 2026

Project Lead: Vaibhavi Yadav

Software Used: Cisco Packet Tracer

1. Project Objective

To design, configure, and troubleshoot a secure Wireless Local Area Network (WLAN) using infrastructure mode, ensuring end-to-end connectivity and robust encryption.

2. Network Topology Components

End Devices: Laptop1, Smartphone

Intermediary Devices: WRT300N Wireless Router, 2960 Switch

Infrastructure: Server-PT (Central Management)

3. Step-by-Step Implementation

Step A: Hardware Interface Upgrade

- The default Laptop1 interface was an Ethernet port, which cannot connect to Wi-Fi.

- Powered OFF Laptop1 in the Physical tab.

- Removed the PT-LAPTOP-NM-1CFE (Ethernet) module.

- Inserted the WPC300N (2.4GHz Wireless) module.

- Powered the laptop back ON.

Step B: Wireless Router Configuration

- SSID Setup: Changed the network name from "Default" to a unique identifier.

- Security Implementation: Enabled WPA2-Personal with AES Encryption.

- LAN Setup: Configured the local IP address to 192.168.0.1 with a Subnet Mask of 255.255.255.0.

- DHCP Management: Disabled the Router's internal DHCP to prevent conflicts with the Server.

Step C: IP Addressing Scheme

Router (Gateway): 192.168.0.1

Server: 192.168.0.2

Laptop1: 192.168.0.5

Subnet Mask: 255.255.255.0 (Class C)

4. Troubleshooting Log (Crucial for Marks)

- IP Typo Correction:

Fixed an error where the Laptop IP was incorrectly set to 192.16.0.5.

Corrected it to 192.168.0.5 to match the gateway subnet.

- Subnet Mask Optimization:

Identified a restrictive subnet mask (255.255.255.252) on the router that was blocking communication.

Corrected this to 255.255.255.0 to allow all devices on the network to communicate.

5. Final Verification

- Physical Connectivity: Verified via dotted lines in the workspace.

- Logical Connectivity: Performed a successful ICMP Ping test from Laptop1 to the Gateway (192.168.0.1).

Results:

Packets Sent = 4

Packets Received = 4

Packets Lost = 0 (0% loss)